AutoDarts Pi Image

Technical Manual & Deployment Guide

Headless Raspberry Pi setup with WiFi captive portal and automatic updates

Version 1.0  •  Raspberry Pi OS Lite 64-bit

By Johan Ä

# Table of Contents

1.  System Overview

2.  Requirements & Hardware

3.  Flashing the Image to SD Card

3.1  Using Raspberry Pi Imager (recommended)

3.2  Using balenaEtcher

3.3  Using dd (Linux/Mac – advanced)

4.  Pre-configuration via autodarts.conf

5.  First Boot – Step by Step

5.1  Boot sequence in detail

5.2  What happens in the background?

6.  WiFi Setup via Captive Portal

6.1  Connecting to AutoDart-Setup

6.2  Filling in the portal

6.3  What happens after saving?

7.  Automatic Updates

8.  System Files & Configuration

9.  Troubleshooting

# 1. System Overview

AutoDarts Pi Image is a pre-built Raspberry Pi installation that allows you to deploy AutoDarts hardware without a monitor, keyboard, or network cable. Everything is configured wirelessly via a built-in captive portal.

| Property | Value |
| --- | --- |
| Base OS | Raspberry Pi OS Lite 64-bit |
| Target Hardware | Raspberry Pi 4 / Raspberry Pi 5 |
| SD Card (min) | 8 GB Class 10 / A1 |
| WiFi AP SSID | AutoDart-Setup |
| Portal Address | http://192.168.4.1 |
| AutoDarts | Installed via get.autodarts.io |
| Auto-update | On every boot (5 min delay), then every 3 days |

## Process Overview

The overall process looks like this:

| 1 | Flash the .img.xz file to an SD card using Raspberry Pi Imager |
| --- | --- |
| 2 | (Optional) Pre-configure WiFi in /boot/firmware/autodarts.conf |
| 3 | Boot the Pi – the system boots headless in 30–90 seconds |
| 4 | Connect your phone/computer to the WiFi network AutoDart-Setup |
| 5 | Enter your WiFi credentials in the portal at 192.168.4.1 |
| 6 | The Pi connects to your WiFi and starts AutoDarts automatically |
| 7 | AutoDarts updates automatically on every boot (after 5 min) and every 3 days |

# 2. Requirements & Hardware

## Required

- Raspberry Pi 4 Model B (2 GB RAM or more) or Raspberry Pi 5

- MicroSD card, minimum 8 GB, recommended 16 GB Class 10 or A1/A2

- USB-C power supply (5V/3A for RPi 4, 5V/5A for RPi 5)

- AutoDarts camera and USB hub (per AutoDarts specifications)

- A computer with an SD card reader to flash the image

## Recommended

- Raspberry Pi Imager installed on your computer (free, from raspberrypi.com/software)

- A phone or computer with WiFi for portal configuration

Note: The Pi does NOT need to be connected to a monitor or keyboard. The entire setup is headless.

# 3. Flashing the Image to SD Card

The image is distributed as a compressed .img.xz file. Raspberry Pi Imager can flash it directly without extracting.

## 3.1  Using Raspberry Pi Imager (recommended)

Raspberry Pi Imager is the easiest method and works on Windows, macOS and Linux.

| 1 | Download and install Raspberry Pi Imager from raspberrypi.com/software |
| --- | --- |
| 2 | Insert the SD card into your computer |
| 3 | Launch Imager and click "Choose OS" |
| 4 | Scroll down and select "Use custom" at the bottom of the list |
| 5 | Select the file autodarts-pi.img.xz (Imager handles .xz natively) |
| 6 | Click "Choose Storage" and select your SD card (be careful – wrong disk will be erased) |
| 7 | Click "Write" and confirm |
| 8 | Wait until the process completes (~5–15 minutes depending on SD card speed) |
| 9 | Safely eject the SD card and insert it into the Pi |

Tip: Raspberry Pi Imager verifies the flash automatically. If verification fails, try a different SD card.

## 3.2  Using balenaEtcher

balenaEtcher is an alternative tool that also works on Windows, macOS and Linux.

| 1 | Download balenaEtcher from etcher.balena.io |
| --- | --- |
| 2 | Open Etcher and click "Flash from file" |
| 3 | Select autodarts-pi.img.xz |
| 4 | Select your SD card under "Select target" |
| 5 | Click "Flash!" and wait for completion |

## 3.3  Using dd (Linux/Mac – advanced)

For advanced users who prefer the command line. Be extra careful to specify the correct disk – choosing the wrong one will permanently erase data.

Decompress and flash in a single command:

xzcat autodarts-pi.img.xz | sudo dd of=/dev/sdX bs=4M status=progress

Replace /dev/sdX with your SD card device (verify with lsblk on Linux or diskutil list on Mac).

# Verify device name on Linux

lsblk

# Verify device name on Mac

diskutil list

Warning: The dd command gives no confirmation dialog. Always double-check that of= points to the SD card and not your hard drive.

# 4. Pre-configuration via autodarts.conf

If you already know your WiFi password, you can skip the portal entirely. Create or edit the file /boot/firmware/autodarts.conf on the SD card before inserting it into the Pi.

The file is read by the system on first boot. The format is:

# /boot/firmware/autodarts.conf

WIFI_SSID=YourNetworkName

WIFI_PASSWORD=YourPassword

AUTODARTS_BOARD_ID=your-board-id

AUTODARTS_API_KEY=your-api-key

| Parameter | Description |
| --- | --- |
| WIFI_SSID | Name of your WiFi network (case-sensitive) |
| WIFI_PASSWORD | WiFi password in plain text |
| AUTODARTS_BOARD_ID | Your AutoDarts Board ID from autodarts.io |
| AUTODARTS_API_KEY | Your API key from autodarts.io |

If the file is missing or empty, the system will start the WiFi portal automatically.

# 5. First Boot – Step by Step

## 5.1  Boot sequence in detail

Here is a timeline from power-on to AutoDarts running:

| 0:00 | Power connected – Pi begins booting |
| --- | --- |
| 0:05 | Bootloader reads SD card, kernel is loaded |
| 0:15 | Linux starts, filesystem is mounted and resized (first boot only) |
| 0:25 | systemd starts all services in parallel |
| 0:30 | setup-ap.service activates – checks autodarts.conf |
| 0:32 | If conf is missing: WiFi AP AutoDart-Setup is started (192.168.4.1) |
| 0:33 | portal.service starts the Flask application on port 80 |
| 0:90 | If conf exists: WiFi connects directly, AutoDarts starts |

## 5.2  What happens in the background?

Here is a detailed description of each systemd service and how they interact.

### setup-ap.service

This service is the heart of the entire setup flow. The script /usr/local/bin/setup-ap.sh runs and does the following:

- Checks whether /boot/firmware/autodarts.conf exists and is populated

- If pre-configuration is found: Configures WiFi using nmcli and proceeds to AutoDarts start

- If no configuration exists: Creates a WiFi access point using NetworkManager

- The AP runs SSID AutoDart-Setup with no password on 192.168.4.1/24

- A built-in timer starts – if nobody connects within 15 minutes, the AP shuts down automatically

### portal.service

The Flask web application at /home/autodart/portal/app.py starts on port 80. It listens for all requests to 192.168.4.1 and presents a web form. The portal handles:

- WiFi SSID and password input

- Optional AutoDarts Board ID and API key input

- Saving configuration to /boot/firmware/autodarts.conf

- Calling nmcli to connect the Pi to the specified network

- Shutting down the AP and restarting the AutoDarts service

Note: If an incorrect WiFi password is entered, it can take up to 30 seconds before a timeout error is shown. This is a known limitation due to how NetworkManager handles authentication failures.

### autodarts.service

The AutoDarts software is installed under /home/autodart and runs as a systemd service. On start:

- The program reads its configuration from /home/autodart/.config/autodarts/config.toml

- It connects to the AutoDarts cloud service using the API key and Board ID

- Camera input is initialized and dart detection begins

### autodarts-updater.service + timer

A systemd timer triggers the updater in two situations: 5 minutes after every boot, and then on a recurring 3-day interval. Each time it runs, it executes curl -sSL https://get.autodarts.io | bash. The update:

- Runs automatically without any user interaction

- Does not interrupt ongoing games (AutoDarts restarts after updating)

- Is logged to the systemd journal (journalctl -u autodarts-updater)

# 6. WiFi Setup via Captive Portal

## 6.1  Connecting to AutoDart-Setup

About 30 seconds after the Pi boots, a new open WiFi network becomes visible:

| Property | Detail |
| --- | --- |
| SSID | AutoDart-Setup |
| Password | (none – open network) |
| Pi IP address | 192.168.4.1 |
| Your IP (DHCP) | 192.168.4.x |

Connect your phone or computer to AutoDart-Setup. Most devices will open a captive portal dialog automatically. If not, open a browser and navigate manually to:

http://192.168.4.1

Tip: Disable mobile data on your phone if the portal doesn't appear – otherwise the phone may route traffic through the cellular network instead.

## 6.2  Filling in the portal

The portal presents a form with the following fields:

- WiFi Network (SSID) – enter the exact name of your home network

- WiFi Password – your network password

- AutoDarts Board ID – found on autodarts.io under your account

- AutoDarts API Key – generated in your AutoDarts account

Click Save / Connect when the form is filled in.

## 6.3  What happens after saving?

| 1 | Form data is sent to the Flask application via HTTP POST |
| --- | --- |
| 2 | app.py saves the credentials to /boot/firmware/autodarts.conf |
| 3 | nmcli is called to configure and connect to your WiFi network |
| 4 | The AutoDart-Setup WiFi AP is shut down |
| 5 | portal.service exits |
| 6 | AutoDarts starts and connects to the cloud service |
| 7 | The Pi is now ready to use on your home network |

After step 4 you will lose your connection to AutoDart-Setup – this is normal and expected. The Pi is now connected to your WiFi.

# 7. Automatic Updates

The system updates AutoDarts automatically on every boot (after a 5-minute delay) and then every 3 days. No manual action is required.

## Checking update status

# View the updater service log

journalctl -u autodarts-updater.service -n 50

# See when the next update is scheduled

systemctl list-timers autodarts-updater.timer

## Manual update

To trigger an update immediately:

sudo systemctl start autodarts-updater.service

AutoDarts will restart automatically after an update. Any game in progress will be interrupted.

# 8. System Files & Configuration

| File / Path | Description |
| --- | --- |
| /usr/local/bin/setup-ap.sh | Main script for WiFi AP management and the setup flow |
| /home/autodart/portal/app.py | Flask captive portal – the web interface |
| /etc/systemd/system/setup-ap.service | systemd service for AP management |
| /etc/systemd/system/portal.service | systemd service for the Flask portal |
| /etc/systemd/system/autodarts-updater.service | systemd service for AutoDarts updates |
| /boot/firmware/autodarts.conf | Pre-configuration file (WiFi, Board ID, API key) |
| /home/autodart/.config/autodarts/config.toml | AutoDarts runtime configuration |

## Checking service status via SSH

Once the Pi is on your network, connect via SSH (default user: autodart):

ssh autodart@<pi-ip-address>

# Check all relevant services

systemctl status setup-ap.service

systemctl status portal.service

systemctl status autodarts.service

systemctl status autodarts-updater.service

# Follow live logs

journalctl -f

# 9. Troubleshooting

| Problem | Possible solution |
| --- | --- |
| AutoDart-Setup not visible | Wait 60–90 seconds. Check that the Pi has power. Restart the Pi. |
| Portal doesn't open automatically | Navigate manually to http://192.168.4.1 in a browser |
| Portal shows error after WiFi input | Check that the password is correct. Timeout takes ~30 sec. |
| Pi doesn't connect to WiFi | Verify SSID (case-sensitive) and password. 5 GHz networks are not supported on RPi 4. |
| AutoDarts doesn't start | Check Board ID and API key in autodarts.conf |
| AP shuts down too early | The AP lives for 15 minutes. Connect quickly after boot, or restart the Pi. |

## Useful logs to check

# System logs since last boot

journalctl -b

# Specific service

journalctl -u setup-ap.service --since today

journalctl -u portal.service --since today

journalctl -u autodarts.service --since today